# --- MAIN RUNNER ---

if __name__ == "__main__":

nodes, elements, fixed_dofs, load_nodes, load_values = setup_truss_ground_structure()

num_elements = len(elements)

areas = np.full(num_elements, INITIAL_AREA)

materials = [{'type': 'steel', 'E': STEEL_E, 'strength': STEEL_STRENGTH}] * num_elements

for i in range(NUM_ITERATIONS):

result = analyze_truss(nodes, elements, areas, materials, fixed_dofs, load_nodes, load_values)

if result[0] is None:

break

displacements, strains, stresses, lengths = result

for j in range(num_elements):

if stresses[j] >= 0:

materials[j] = {'type': 'steel', 'E': STEEL_E, 'strength': STEEL_STRENGTH}

else:

materials[j] = {'type': 'cfrp', 'E': CFRP_E, 'strength': CFRP_STRENGTH}

target_areas = np.abs(stresses) / np.array([m['strength'] for m in materials]) * areas

temperature = 1.0 - (i / NUM_ITERATIONS)

random_factor = 1.0 + (np.random.rand(num_elements) - 0.5) * 0.1 * temperature

new_areas = target_areas * random_factor

if i > COMPLEXITY_CONTROL_START_ITER:

max_area = np.max(areas)

for j in range(num_elements):

if areas[j] < 0.05 * max_area and np.abs(stresses[j]) < 0.5 * materials[j]['strength']:

new_areas[j] *= 0.7

areas = np.clip(new_areas, MIN_AREA, None)

final = analyze_truss(nodes, elements, areas, materials, fixed_dofs, load_nodes, load_values)

if final[0] is not None:

export_for_karamba(nodes, elements, areas, materials)