### 5. format_results_as_json

python

def format_results_as_json(nodes, elements, areas, stresses, element_lengths):

"""Formats results as JSON-serializable dictionary.

Returns:

dict: Optimized truss data ready for JSON serialization

"""

active_mask = areas > MIN_AREA * 1.1

total_volume_m3 = np.sum(areas[active_mask] * element_lengths[active_mask]) / 1e9

total_mass_kg = total_volume_m3 * (STEEL_DENSITY * 1e9)

member_data = []

for i, (n1_idx, n2_idx) in enumerate(elements):

if not active_mask[i]: continue

stress = stresses[i]

force_kN = (stress * areas[i]) / 1000.0

status = "Tension" if stress >= 0 else "Compression"

start_node = {'x': round(nodes[n1_idx, 0]/1000, 3),

'y': round(nodes[n1_idx, 1]/1000, 3)}

end_node = {'x': round(nodes[n2_idx, 0]/1000, 3),

'y': round(nodes[n2_idx, 1]/1000, 3)}

member_data.append({

'memberId': i,

'startNode': start_node,

'endNode': end_node,

'length_m': round(element_lengths[i]/1000.0, 3),

'area_cm2': round(areas[i]/100.0, 2),

'stress_MPa': round(stress, 2),

'force_kN': round(force_kN, 2),

'status': status

})

return {

"summary": {

"totalActiveMembers": len(member_data),

"totalVolume_m3": round(total_volume_m3, 4),

"totalMass_kg": round(total_mass_kg, 2)

},

"members": member_data

}

Metadata:

- Dependencies: numpy, json, Globals: MIN_AREA, STEEL_DENSITY

- Inputs:

  - nodes: Node coordinates

  - elements: Member connections

  - areas: Final member areas

  - stresses: Final member stresses

  - element_lengths: Member lengths

- Output: Dictionary containing truss summary and member details

Description:
Prepares final results for output by:

- Filtering active members (>1.1×MIN_AREA)

- Calculating total volume and mass

- Converting units (mm → m, mm² → cm², N → kN)

- Packaging member data with coordinates

- Structuring JSON-ready output