Reference_Code

Note: Here are libraries used, you cant use in your script.

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.collections import LineCollection

from matplotlib.lines import Line2D

import math

import csv

# --- PROBLEM DEFINITION & PARAMETERS ---

SPAN = 20000  # mm

HEIGHT = 2500  # mm

NUM_BAYS = 10

POINT_LOAD = 1000000  # N

# --- MATERIAL PROPERTIES ---

STEEL_E = 2.0e5

STEEL_STRENGTH = 355

CFRP_E = 1.5e5

CFRP_STRENGTH = 300

# --- OPTIMIZATION PARAMETERS ---

INITIAL_AREA = 5000

MIN_AREA = 10

NUM_ITERATIONS = 150

COMPLEXITY_CONTROL_START_ITER = 75

# --- TRUSS SETUP ---

def setup_truss_ground_structure():

nodes = []

for i in range(NUM_BAYS + 1):

x = i * (SPAN / NUM_BAYS)

nodes.append([x, 0])

nodes.append([x, HEIGHT])

nodes = np.array(nodes)

elements = []

num_nodes_total = len(nodes)

for i in range(num_nodes_total):

for j in range(i + 1, num_nodes_total):

if abs(nodes[i][0] - nodes[j][0]) <= 2 * (SPAN / NUM_BAYS):

elements.append([i, j])

elements = np.array(elements)

bottom_right_node_idx = 2 * NUM_BAYS

fixed_dofs = [0, 1, 2 * bottom_right_node_idx + 1]

middle_node_index = NUM_BAYS + 1

load_nodes = [middle_node_index]

load_values = np.array([[0, -POINT_LOAD]])

return nodes, elements, fixed_dofs, load_nodes, load_values

# --- FEA FUNCTION ---

def analyze_truss(nodes, elements, areas, materials, fixed_dofs, load_nodes, load_values):

num_nodes = len(nodes)

num_elements = len(elements)

num_dofs = num_nodes * 2

K = np.zeros((num_dofs, num_dofs))

element_lengths = np.zeros(num_elements)

for i, (n1_idx, n2_idx) in enumerate(elements):

n1, n2 = nodes[n1_idx], nodes[n2_idx]

L = np.linalg.norm(n2 - n1)

if L == 0:

continue

element_lengths[i] = L

c = (n2[0] - n1[0]) / L

s = (n2[1] - n1[1]) / L

cc, cs, ss = c * c, c * s, s * s

E = materials[i]['E']

A = areas[i]

k_e = (A * E / L) * np.array([

[cc, cs, -cc, -cs],

[cs, ss, -cs, -ss],

[-cc, -cs, cc, cs],

[-cs, -ss, cs, ss]

])

dofs = [2 * n1_idx, 2 * n1_idx + 1, 2 * n2_idx, 2 * n2_idx + 1]

for row, dof_r in enumerate(dofs):

for col, dof_c in enumerate(dofs):

if dof_r < num_dofs and dof_c < num_dofs:

K[dof_r, dof_c] += k_e[row, col]

F = np.zeros(num_dofs)

for i, node_idx in enumerate(load_nodes):

dof_x, dof_y = 2 * node_idx, 2 * node_idx + 1

F[dof_x] += load_values[i, 0]

F[dof_y] += load_values[i, 1]

free_dofs = np.setdiff1d(np.arange(num_dofs), fixed_dofs)

U = np.zeros(num_dofs)

try:

if len(free_dofs) > 0:

U[free_dofs] = np.linalg.solve(K[np.ix_(free_dofs, free_dofs)], F[free_dofs])

except np.linalg.LinAlgError:

print("Singular matrix — structure unstable.")

return None, None, None, None

strains = np.zeros(num_elements)

for i, (n1_idx, n2_idx) in enumerate(elements):

L = element_lengths[i]

if L == 0:

continue

n1, n2 = nodes[n1_idx], nodes[n2_idx]

c = (n2[0] - n1[0]) / L

s = (n2[1] - n1[1]) / L

u1x, u1y = U[2 * n1_idx], U[2 * n1_idx + 1]

u2x, u2y = U[2 * n2_idx], U[2 * n2_idx + 1]

strains[i] = (1 / L) * (c * (u2x - u1x) + s * (u2y - u1y))

stresses = np.array([materials[i]['E'] * strains[i] for i in range(num_elements)])

return U.reshape((num_nodes, 2)), strains, stresses, element_lengths

# --- VISUALIZATION ---

def plot_truss(ax, nodes, elements, areas, materials, title, displacements=None):

ax.cla()

plot_nodes = nodes + np.nan_to_num(displacements * 20) if displacements is not None else nodes

max_area = max(1, np.max(areas))

widths = 10 * (areas / max_area)

colors = ['royalblue' if m['type'] == 'steel' else 'firebrick' for m in materials]

lines = [[plot_nodes[e[0]], plot_nodes[e[1]]] for e in elements]

lc = LineCollection(lines, linewidths=widths, colors=colors, zorder=2)

ax.add_collection(lc)

ax.scatter(nodes[:, 0], nodes[:, 1], c='black', s=20, zorder=3)

ax.set_title(title)

ax.set_aspect('equal')

ax.grid(True)

# --- EXPORT TO CSV ---

def export_for_karamba(nodes, elements, areas, materials, filename="karamba_truss_export.csv"):

print(f"Exporting final truss data to {filename}...")

with open(filename, 'w', newline='') as f:

writer = csv.writer(f)

writer.writerow([

'MemberID', 'StartX', 'StartY', 'StartZ',

'EndX', 'EndY', 'EndZ', 'Material',

'Area_cm2', 'ExampleDiameter_mm'

])

for i, (n1, n2) in enumerate(elements):

if areas[i] <= MIN_AREA * 1.1:

continue

start = nodes[n1] / 1000.0

end = nodes[n2] / 1000.0

mat = 'Steel_Tension' if materials[i]['type'] == 'steel' else 'CFRP_Compression'

area_cm2 = areas[i] / 100

diameter_mm = 2 * math.sqrt(areas[i] / math.pi)

writer.writerow([

f"member_{i}", start[0], start[1], 0.0,

end[0], end[1], 0.0, mat,

f"{area_cm2:.2f}", f"{diameter_mm:.2f}"

])

print("Export complete.")

# --- MAIN RUNNER ---

if __name__ == "__main__":

nodes, elements, fixed_dofs, load_nodes, load_values = setup_truss_ground_structure()

num_elements = len(elements)

areas = np.full(num_elements, INITIAL_AREA)

materials = [{'type': 'steel', 'E': STEEL_E, 'strength': STEEL_STRENGTH}] * num_elements

plt.ion()

for i in range(NUM_ITERATIONS):

result = analyze_truss(nodes, elements, areas, materials, fixed_dofs, load_nodes, load_values)

if result[0] is None:

break

displacements, strains, stresses, lengths = result

for j in range(num_elements):

if stresses[j] >= 0:

materials[j] = {'type': 'steel', 'E': STEEL_E, 'strength': STEEL_STRENGTH}

else:

materials[j] = {'type': 'cfrp', 'E': CFRP_E, 'strength': CFRP_STRENGTH}

target_areas = np.abs(stresses) / np.array([m['strength'] for m in materials]) * areas

temperature = 1.0 - (i / NUM_ITERATIONS)

random_factor = 1.0 + (np.random.rand(num_elements) - 0.5) * 0.1 * temperature

new_areas = target_areas * random_factor

if i > COMPLEXITY_CONTROL_START_ITER:

max_area = np.max(areas)

for j in range(num_elements):

if areas[j] < 0.05 * max_area and np.abs(stresses[j]) < 0.5 * materials[j]['strength']:

new_areas[j] *= 0.7

areas = np.clip(new_areas, MIN_AREA, None)

if i % 5 == 0 or i == NUM_ITERATIONS - 1:

fig, ax = plt.subplots(figsize=(16, 5))

plot_truss(ax, nodes, elements, areas, materials, f"Iteration {i+1}/{NUM_ITERATIONS}")

plt.tight_layout()

plt.show()

plt.ioff()

final = analyze_truss(nodes, elements, areas, materials, fixed_dofs, load_nodes, load_values)

if final[0] is not None:

final_disp = final[0]

fig_final, ax_final = plt.subplots(figsize=(16, 5))

plot_truss(ax_final, nodes, elements, areas, materials, "Final Optimized Truss Design", final_disp)

ax_final.legend([

Line2D([0], [0], color='royalblue', lw=4, label='Tension (Steel)'),

Line2D([0], [0], color='firebrick', lw=4, label='Compression (CFRP)')

])

plt.show()

export_for_karamba(nodes, elements, areas, materials)