This is an example script containing essential components and structure.

import numpy as np

# SECTION 1: INPUT PARAMETERS

# --- TRUSS GEOMETRY & DOMAIN ---

SPAN = 15000  # 15 meters in mm

HEIGHT = 3000  # 3 meters in mm

NUM_BAYS = 12

# --- LOADING CONDITIONS ---

FACTORED_LINE_LOAD_PER_MM = 750000 / (NUM_BAYS + 1)  # Total load distributed across the bay width

# --- MATERIAL PROPERTIES ---

ALUMINUM_E = 70000.0  # Modulus of Elasticity for Aluminum in MPa (N/mm^2)

CFRP_E = 230000.0     # Modulus of Elasticity for CFRP in MPa (N/mm^2)

ALUMINUM_STRENGTH_TENSION = 200.0  # Allowable tensile stress in MPa for Aluminum

CFRP_STRENGTH_COMPRESSION = 300.0  # Allowable compressive stress in MPa for CFRP

ALUMINUM_DENSITY = 2.7e-6  # kg/mm^3 for Aluminum

CFRP_DENSITY = 1.6e-6      # kg/mm^3 for CFRP

# --- OPTIMIZATION PARAMETERS ---

INITIAL_AREA = 20000

MIN_AREA = 1.0

NUM_ITERATIONS = 120

EVOLUTIONARY_RATE = 0.03

MAX_STRESS_ADJUSTMENT_RATE = 0.5

# --- BUCKLING PARAMETERS ---

T_D_RATIO = 0.1

K_FACTOR_BUCKLING = 1.0

# SECTION 2: SETUP AND STRUCTURAL ANALYSIS

def setup_truss_ground_structure():

"""Creates the initial dense mesh of nodes and potential elements (ground structure)."""

nodes = []

for i in range(NUM_BAYS + 1):

x = i * (SPAN / NUM_BAYS)

nodes.append([x, 0])      # Bottom chord node

nodes.append([x, HEIGHT]) # Top chord node

nodes = np.array(nodes)

elements, chord_indices, web_indices = [], [], []

num_nodes_total = len(nodes)

element_counter = 0

for i in range(num_nodes_total):

for j in range(i + 1, num_nodes_total):

node1_x, node1_y = nodes[i]

node2_x, node2_y = nodes[j]

if abs(node1_x - node2_x) <= 3 * (SPAN / NUM_BAYS):

elements.append([i, j])

is_chord = (node1_y == node2_y) and (abs(node1_x - node2_x) <= (SPAN / NUM_BAYS) * 1.01)

if is_chord:

chord_indices.append(element_counter)

else:

web_indices.append(element_counter)

element_counter += 1

elements = np.array(elements)

bottom_right_node_idx = 2 * NUM_BAYS

fixed_dofs = [

0,

1,

2 * bottom_right_node_idx + 1,

]

load_nodes = [1]

load_values = np.array([[0, -FACTORED_LINE_LOAD_PER_MM]])

return nodes, elements, fixed_dofs, np.array(load_nodes), load_values, np.array(chord_indices), np.array(web_indices)

def analyze_truss(nodes, elements, areas, fixed_dofs, external_loads):

"""Performs Finite Element Analysis (FEA) including self-weight."""

num_nodes = len(nodes)

num_elements = len(elements)

num_dofs = num_nodes * 2

K = np.zeros((num_dofs, num_dofs))

element_lengths = np.zeros(num_elements)

self_weight_forces = np.zeros((num_nodes, 2))

for i, (n1_idx, n2_idx) in enumerate(elements):

L = np.linalg.norm(nodes[n2_idx] - nodes[n1_idx])

element_lengths[i] = L

member_weight = areas[i] * L * ALUMINUM_DENSITY * 9.81 / 1000.0

self_weight_forces[n1_idx, 1] -= member_weight / 2.0

self_weight_forces[n2_idx, 1] -= member_weight / 2.0

for i, (n1_idx, n2_idx) in enumerate(elements):

L = element_lengths[i]

if L == 0:

continue

c, s = (nodes[n2_idx, 0] - nodes[n1_idx, 0]) / L, (nodes[n2_idx, 1] - nodes[n1_idx, 1]) / L

A = areas[i]

k_e = (A * ALUMINUM_E / L) * np.array([

[c * c, c * s, -c * c, -c * s],

[c * s, s * s, -c * s, -s * s],

[-c * c, -c * s, c * c, c * s],

[-c * s, -s * s, c * s, s * s]

])

dofs = [2 * n1_idx, 2 * n1_idx + 1, 2 * n2_idx, 2 * n2_idx + 1]

K[np.ix_(dofs, dofs)] += k_e

F = external_loads.flatten() + self_weight_forces.flatten()

free_dofs = np.setdiff1d(range(num_dofs), fixed_dofs)

U = np.zeros(num_dofs)

try:

U[free_dofs] = np.linalg.solve(K[np.ix_(free_dofs, free_dofs)], F[free_dofs])

except np.linalg.LinAlgError:

return None, None, None, None

strains = np.zeros(num_elements)

for i, (n1_idx, n2_idx) in enumerate(elements):

L = element_lengths[i]

if L == 0:

continue

c, s = (nodes[n2_idx, 0] - nodes[n1_idx, 0]) / L, (nodes[n2_idx, 1] - nodes[n1_idx, 1]) / L

u1x, u1y = U[2 * n1_idx], U[2 * n1_idx + 1]

u2x, u2y = U[2 * n2_idx], U[2 * n2_idx + 1]

strains[i] = (c * (u2x - u1x) + s * (u2y - u1y)) / L

stresses = ALUMINUM_E * strains

return U.reshape((num_nodes, 2)), strains, stresses, element_lengths

def calculate_buckling_stress(areas, lengths):

"""Calculates the critical buckling stress for each member."""

safe_areas = np.maximum(areas, MIN_AREA)

I = safe_areas ** 2 / (8 * np.pi * T_D_RATIO)

effective_lengths = K_FACTOR_BUCKLING * lengths + 1e-9

critical_stress = (np.pi ** 2 * CFRP_E * I) / (effective_lengths ** 2 * safe_areas)

return np.minimum(critical_stress, CFRP_STRENGTH_COMPRESSION)

def update_areas_beso_with_chords(areas, stresses, element_lengths, chord_indices, web_indices):

"""Updates areas using BESO, with materials considered for buckling."""

new_areas = areas.copy()

allowable_compressive_stress = calculate_buckling_stress(areas, element_lengths)

allowable_stresses = np.where(stresses >= 0, ALUMINUM_STRENGTH_TENSION, allowable_compressive_stress)

allowable_stresses[allowable_stresses < 1e-3] = 1.0

stress_ratio = np.abs(stresses) / allowable_stresses

target_areas = areas * stress_ratio

new_areas = (1 - MAX_STRESS_ADJUSTMENT_RATE) * areas + MAX_STRESS_ADJUSTMENT_RATE * target_areas

return np.maximum(MIN_AREA, new_areas)

def export_final_design_to_json(nodes, elements, areas):

"""Exports the final truss design data as a list of dictionaries."""

results = []

for i, (n1_idx, n2_idx) in enumerate(elements):

if areas[i] <= MIN_AREA * 1.1:

continue

data = {

"member_id": f"member_{i + 1:02d}",

"start_x": nodes[n1_idx][0] / 1000.0,

"start_y": nodes[n1_idx][1] / 1000.0,

"end_x": nodes[n2_idx][0] / 1000.0,

"end_y": nodes[n2_idx][1] / 1000.0,

"area": areas[i] / 100.0

}

results.append(data)

return results

# SECTION 5: MAIN EXECUTION SCRIPT

def generate_truss_design():

# Setup truss structure

nodes, elements, fixed_dofs, load_nodes, load_values, chord_indices, web_indices = setup_truss_ground_structure()

num_elements = len(elements)

areas = np.full(num_elements, INITIAL_AREA, dtype=float)

# Apply loads

external_loads = np.zeros((len(nodes), 2))

for i, node_idx in enumerate(load_nodes):

external_loads[node_idx, :] += load_values[i, :]

# Run optimization

for i in range(NUM_ITERATIONS):

analysis_results = analyze_truss(nodes, elements, areas, fixed_dofs, external_loads)

if analysis_results[0] is None:

break

displacements, strains, stresses, element_lengths = analysis_results

areas = update_areas_beso_with_chords(areas, stresses, element_lengths, chord_indices, web_indices)

# Prepare final output

final_results = export_final_design_to_json(nodes, elements, areas)

return final_results  # Explicit return of the array of dictionaries

# This is the output that n8n will receive

output_data = generate_truss_design()

return output_data