### 2. analyze_truss

python

def analyze_truss(nodes, elements, areas, fixed_dofs, external_loads):

"""Performs Finite Element Analysis (FEA) including self-weight.

Returns:

tuple: (displacements, strains, stresses, element_lengths) or (None, None, None, None) on failure

"""

num_nodes, num_elements, num_dofs = len(nodes), len(elements), len(nodes) * 2

K = np.zeros((num_dofs, num_dofs))

element_lengths = np.zeros(num_elements)

self_weight_forces = np.zeros((num_nodes, 2))

for i, (n1_idx, n2_idx) in enumerate(elements):

L = np.linalg.norm(nodes[n2_idx] - nodes[n1_idx])

element_lengths[i] = L

member_weight = areas[i] * L * STEEL_DENSITY * 9.81 / 1000.0

self_weight_forces[n1_idx, 1] -= member_weight / 2.0

self_weight_forces[n2_idx, 1] -= member_weight / 2.0

if L == 0: continue

c, s = (nodes[n2_idx,0] - nodes[n1_idx,0]) / L, (nodes[n2_idx,1] - nodes[n1_idx,1]) / L

k_e = (areas[i] * STEEL_E / L) * np.array([[c*c, c*s, -c*c, -c*s],

[c*s, s*s, -c*s, -s*s],

[-c*c, -c*s, c*c, c*s],

[-c*s, -s*s, c*s, s*s]])

dofs = [2*n1_idx, 2*n1_idx+1, 2*n2_idx, 2*n2_idx+1]

K[np.ix_(dofs, dofs)] += k_e

F = external_loads.flatten() + self_weight_forces.flatten()

free_dofs = np.setdiff1d(range(num_dofs), fixed_dofs)

U = np.zeros(num_dofs)

try:

U[free_dofs] = np.linalg.solve(K[np.ix_(free_dofs, free_dofs)], F[free_dofs])

except np.linalg.LinAlgError:

return None, None, None, None

strains = np.zeros(num_elements)

for i, (n1_idx, n2_idx) in enumerate(elements):

L = element_lengths[i]

if L == 0: continue

c, s = (nodes[n2_idx,0] - nodes[n1_idx,0])/L, (nodes[n2_idx,1] - nodes[n1_idx,1])/L

strains[i] = (c * (U[2*n2_idx] - U[2*n1_idx]) + s * (U[2*n2_idx+1] - U[2*n1_idx+1])) / L

stresses = STEEL_E * strains

return U.reshape((num_nodes, 2)), strains, stresses, element_lengths

Metadata:

- Dependencies: numpy, Global constants: STEEL_E, STEEL_DENSITY

- Inputs:

  - nodes: Node coordinates

  - elements: Member connections

  - areas: Cross-sectional areas of members

  - fixed_dofs: Constrained degrees of freedom

  - external_loads: Applied nodal loads

- Outputs: Displacements, strains, stresses, and member lengths

Description:
Performs linear static FEA by:

- Building global stiffness matrix

- Calculating self-weight forces

- Solving displacement equations

- Computing member strains and stresses

- Handling singular matrix errors (unstable structures)