### 3. calculate_buckling_stress

python

def calculate_buckling_stress(areas, lengths):

"""Calculates critical buckling stress using Euler's formula.

Returns:

np.array: Allowable compressive stresses for each member

"""

safe_areas = np.maximum(areas, MIN_AREA)

I = safe_areas**2 / (8 * np.pi * T_D_RATIO)

effective_lengths = K_FACTOR_BUCKLING * lengths + 1e-9

critical_stress = (np.pi**2 * STEEL_E * I) / (effective_lengths**2 * safe_areas)

return np.minimum(critical_stress, STEEL_STRENGTH_COMPRESSION)

Metadata:

- Dependencies: numpy, math, Globals: MIN_AREA, T_D_RATIO, K_FACTOR_BUCKLING, STEEL_E, STEEL_STRENGTH_COMPRESSION

- Inputs:

  - areas: Member cross-sectional areas

  - lengths: Member lengths

- Output: Array of allowable compressive stresses

Description:
Computes buckling resistance for compression members by:

- Approximating moment of inertia from area

- Calculating Euler buckling stress

- Capping stress at material compressive strength

- Adding epsilon to avoid division by zero