### 1. setup_truss_ground_structure

python

def setup_truss_ground_structure():

"""

Creates the initial dense mesh of nodes and potential elements (ground structure).

Returns:

tuple: (nodes, elements, fixed_dofs, load_nodes, load_values, chord_indices, web_indices)

"""

nodes = []

for i in range(NUM_BAYS + 1):

x = i * (SPAN / NUM_BAYS)

nodes.append([x, 0])

nodes.append([x, HEIGHT])

nodes = np.array(nodes)

elements, chord_indices, web_indices = [], [], []

element_counter = 0

for i in range(len(nodes)):

for j in range(i + 1, len(nodes)):

node1_x, node1_y = nodes[i]

node2_x, node2_y = nodes[j]

if abs(node1_x - node2_x) <= 3 * (SPAN / NUM_BAYS):

elements.append([i, j])

is_chord = (node1_y == node2_y) and (abs(node1_x - node2_x) <= (SPAN / NUM_BAYS) * 1.01)

if is_chord:

chord_indices.append(element_counter)

else:

web_indices.append(element_counter)

element_counter += 1

elements = np.array(elements)

bottom_right_node_idx = 2 * NUM_BAYS

fixed_dofs = [0, 1, 2 * bottom_right_node_idx + 1]

load_nodes, load_values = [], []

bay_width = SPAN / NUM_BAYS

for i in range(1, len(nodes), 2):

load_nodes.append(i)

load = -FACTORED_LINE_LOAD_PER_MM * bay_width

if i == 1 or i == (len(nodes) - 1):

load /= 2

load_values.append([0, load])

return nodes, elements, fixed_dofs, np.array(load_nodes), np.array(load_values), np.array(chord_indices), np.array(web_indices)

Metadata:

- Dependencies: numpy (as np), Global constants: SPAN, HEIGHT, NUM_BAYS, FACTORED_LINE_LOAD_PER_MM

- Inputs: None

- Outputs:

  - nodes: Array of node coordinates [x,y] in mm

  - elements: Array of member connections [node_i, node_j]

  - fixed_dofs: List of constrained degrees of freedom

  - load_nodes: Indices of loaded nodes

  - load_values: Corresponding load vectors [Fx, Fy]

  - chord_indices: Indices of chord members

  - web_indices: Indices of web members

Description:
Generates the initial truss topology by:

- Creating nodes along top and bottom chords

- Connecting nodes within 3-bay proximity

- Classifying members as chords (horizontal) or webs (diagonal/vertical)

- Applying boundary conditions (left fixed, right roller)

- Calculating nodal loads from distributed load