### 4. update_areas_beso_with_chords

python

def update_areas_beso_with_chords(areas, stresses, element_lengths, chord_indices, web_indices):

"""Updates areas using BESO method with special chord handling.

Returns:

np.array: Updated member areas

"""

allowable_compressive_stress = calculate_buckling_stress(areas, element_lengths)

allowable_stresses = np.where(stresses >= 0, STEEL_STRENGTH_TENSION, allowable_compressive_stress)

allowable_stresses[allowable_stresses < 1e-3] = 1.0

stress_ratio = np.abs(stresses) / allowable_stresses

new_areas = (1 - MAX_STRESS_ADJUSTMENT_RATE) * areas + MAX_STRESS_ADJUSTMENT_RATE * (areas * stress_ratio)

web_areas = new_areas[web_indices]

web_stresses = stresses[web_indices]

web_lengths = element_lengths[web_indices]

target_web_volume = np.sum(web_areas * web_lengths) * (1 - EVOLUTIONARY_RATE)

sensitivity = np.abs(web_stresses)

sorted_web_indices_by_sensitivity = web_indices[np.argsort(sensitivity)]

current_web_volume = np.sum(new_areas[web_indices] * web_lengths)

for idx in sorted_web_indices_by_sensitivity:

if current_web_volume <= target_web_volume: break

volume_of_element = (new_areas[idx] - MIN_AREA) * element_lengths[idx]

if volume_of_element > 0:

new_areas[idx] = MIN_AREA

current_web_volume -= volume_of_element

return np.maximum(MIN_AREA, new_areas)

Metadata:

- Dependencies: numpy, calculate_buckling_stress, Globals: STEEL_STRENGTH_TENSION, MIN_AREA, EVOLUTIONARY_RATE, MAX_STRESS_ADJUSTMENT_RATE

- Inputs:

  - areas: Current member areas

  - stresses: Calculated member stresses

  - element_lengths: Member lengths

  - chord_indices: Indices of chord members

  - web_indices: Indices of web members

- Output: Updated member areas after optimization

Description:
Implements BESO optimization by:

- Calculating stress utilization ratios

- Adjusting areas based on stress levels

- Applying evolutionary volume reduction to web members

- Removing low-stress web members first

- Enforcing minimum area constraints